- Milestone 1: Fill Head

  - 1a. Separate tool heads

    - Caps are separate from pouches

    - Pros:

      - Does not require cap alignment

      - Tool heads only move on the z axis

      - Allows for production pipeline

    - Cons:

      - Requires more vertical actuators

      - Requires user to take off cap from pouch before loading

      - Requires that the caps be stored somehow to be later sealed on the pouch

  - 1b. Combined tool head

    - Linear vs Rotating

    - Caps attached to loaded pouches

    - Pros:

      - Singular vertical actuator needed

      - Less work for the user (don’t have to take cap off before loading)

    - Cons:

      - Only one operation can happen at a time

      - Must move in minimum of 2 axis (or 1 axis and 1 circle)

      - Requires cap alignment

- Milestone 2: Cap Alignment/Feeder

  - 2a. Vibration into form fit hole

    - Uses air or vibration to shake caps into form fit holes

    - Pros:

      - Cap may be picked up in any orientation

      - Cap may be on or off of pouch

    - Cons:

      - Relies on random processes

      - Possible jams

  - 2b. Rotating grabber

    - For cap on pouch

    - Attempts to grab the cap while rotating the clamp until the clamp is closed around the thinnest section of the cap

    - Pros:

      - Alignment is done before cap is grabbed

    - Cons:

      - Requires ability to measure clamp opening distance (complex)

      - Requires clamping actuation and rotation (2 degrees of freedom)

  - 2c. Separate Caps

    - For cap off pouch

    - Uses 2 rails for cap storage

    - Pros:

      - Does not require alignment

      - No additional degrees of freedom needed

    - Cons:

      - Can only be used with separate fill heads

      - User must place caps into storage device

- Milestone 3: Pouch Movement

  - 3a. Belt Fed

    - Uses a belt to advance pouches through the machine

    - Pros:

      - May use teeth or clamps (current securing method) to hold pouches

      - Straight lines for mounting

    - Cons:

      - More moving pieces

      - Slack in belt could be a source of error

  - 3b. Disk Fed (Merry-Go-Round)

    - Pouches secured to rotating disk

    - Pros:

      - Fixed positioning (using rotation)

      - Less moving parts

    - Cons:

      - Pouches must use clamps (current securing method) to be held in place

      - Design is circular (mounting is harder)

- Milestone 4: Pouch Loading

  - 4a. Manual

    - Pros:

      - Reduced complexity

    - Cons:

      - Less automated/Requires user interaction

  - 4b. Vertical Cartridge

    - Uses hook to grab pouches

    - Pros:

      - Compact

    - Cons:

      - Hook may catch/damage pouches

      - Pouches cannot have caps on

  - 4c. Slanted Cartridge

    - Uses roller to grab pouches

    - Pros:

      - Pouches can have caps or no caps

      - Rollers allow precise positioning

    - Cons:

      - Requires longer loading ramp